| Document Change History |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Date | Ver. | Editor | Chap. | Description | CR# |
| 2018.11.30 | 1.0 | 이재혁 | ALL | Initial Creation | SCR-181101-005 |
| 2019.02.01 | 2.0 | 이재혁 | 4.2.x 4.3.x 4.4.x 4.6.1 | Read Data by Identifier (22 hex) 항목 추가 – 2세대/3세대 구분 강제 구동/DTC Reset Condtion 전원 조건 변경 | SCR-190121-006 |
| 2019.04.26 | 3.0 | 이재혁 | 4.6.1 | Local CAN 관련 DTC 코드 수정 (DIMS 마스터 DB 코드와 매칭 필요) B-CAN TIMEOUT Detecting time 수정 | SCR-190528-002 |
| 2019.07.22 | 4.0 | 이재혁 | 4.2.8.1 4.5.1 4.6.1 | Charging Status Bit 오류 수정 Report DTC Extended Data Record By DTC Number 내용 삭제 코일 불량 조건에서 DTC 미출력 | SCR-190722-005 SCR-190722-004 |
| 2019.12.23 | 4.1 | 이재혁 | 4.6.1 | 고속 B-CAN에서는 B1602 DTC 미적용 명기 | SCR-191223-005 |
| 2020.01.23 | 4.2 | 이재혁 | 4.2.7 | 상황별 Active Coil ID 출력방안 명확화 | SCR-200123-001 |
| 2020.04.03 | 4.3 | 이재혁 | 4.2.9 | RDBI 항목 추가 및 내용 보완 - F1A0, F1A1, F1B0, F1B1, F191, F193 추가 및 수정 | SCR-200406-010 |
| 2020.06.04 | 4.4 | 이재혁 | 4.2.9 | RDBI Data Size 오류 수정 - F1A1, F1B0, F1B1, F191 RDBI Description 오기 내용 수정 | SCR-200519-004 |
| 2021.05.24 | 4.5 | 이재혁 | 4.2.9 | RDBI 항목 추가 - F1EF (RXSWIN 대응 추가) | SCR-210524-007 |
| 2021.08.24 | 4.6 | 이재혁 | 4.2.8 | DID 버전 표기 방법 변경 (3Byte -> 4Byte / ”.” 추가) - F1A0, F1B1, F191, F193, F195, F100 | SCR-210824-003 |
| 2021.12.08 | 4.7 | 조길상 | 4.2.9 | RDBI 항목 추가 - F1C1 (SW무결성검증 추가) | SCR-211108-003 |
| 2022.08.08 | 4.8 | 조길상 | 4.2.9 | DID 항목 및 표기법 통일 - 제어기 진단 DID 코드 표준화안 적용 (F1A0, F195 삭제) | TSCR-143 |
| 2022.08.08 | 4.9 | 조길상 | 4.2.9 | WPC 플랫폼 버전 출력 DID 추가(F17F,F17D 추가) | TSCR-238 |
| 2023.02.20 | 5.0 | 조길상 | 4.6.1 | 제어기 로그 저장 기능 지원을 위한 EC (Extended DTC Code) 추가 - DTC Definition for WPC ECU 수정 | TSCR-734 |
| 2023.02.20 | 5.1 | 조길상 | 4.1.2 | WPC,WPC2,RWPC 진단 ID 오기 수정 | TSCR-759 |
| 2023.07.11 | 5.2 | 조길상 | 4.2.8.2 4.3.2 4.6.1 | 1. GREEN IND 모니터링 사양 명확화 - Animation 동작중 On 출력 2. GREEN IND 강제 출력 사양 명확화 - 3개 LED 모두 동작 3. 내부고장 DTC 사양 Description 명확화 | TSCR-1280 |
| 2023.10.26 | 5.3 | 조길상 | 4.3.2 | 강제구동 Input Output Control Parameter 수정 | TSCR-1751 |
| 2023.12.13 | 5.4 | 조길상 | 4.2.9.15 | DID ED90 - NonSleepCollectedEachEcuDataIdentifie추가 | TSCR-1713 |
| 2024.06.14 | 5.5 | 조길상 | 4.2.7.1 4.2.8.1 4.3.2 4.2.9.13 | 1. 입출력 모니터링 듀얼 사양 추가 2. 강제구동 듀얼 사양 추가 3. 플랫폼 버전 표시 방법 변경 (11byte 14byte) | YSCR-201 |
| 2024.12.13 | 5.6 | 정은영 | 4.2.7 4.2.9.4 | 1. DID BE02 - WPC2 FAN Speed 추가 2. 차종 소프트웨어 버전 표시 방법 변경 (4byte 5byte) - F1B1 | YSCR-220 |
| 2025.02.14 | 5.7 | 정은영 | 4.2.7.X 4.2.8.X 4.2.5 4.2.9 | 1. DID BE02 – WPC2 COIL TempSensor 명칭 명확화 2. DID BE03 – EPP 모드 인증 기능 명확화 3. DID F1C0 – HSM 현재 상태값 정보 출력 추가 | YSCR-252 |
| 2025.04.24 | 5.8 | 곽승훈 | 4.3.2 | 1. 강제구동 항목 F014~F01E 추가 | YSCR-270 |

목 차

1	목적	4

2	적용범위	4

3	용어정의	4

4	DIAGNOSTIC SPECIFICATION	6

4.1	Protocol Definition	6

4.1.1	진단 통신 서비스 지원 목록	6

4.1.2	Communication Parameters	6

4.1.3	DiagnosticSessionControl (10 hex) Service	7

4.1.4	SecurityAccess(27 hex) Service	7

4.1.4.1	Request Message	7

4.1.4.2	Positive Response Message	7

4.1.4.3	Supported Negative Response Codes	7

4.1.4.4	Security Algorithm (ASK not Applied)	7

4.1.4.5	Security Algorithm (ASK Applied)	8

4.1.5	StopDiagnosticSession (20 hex) Service	8

4.2	Read Data by Identifier (22 hex) Service	9

4.2.1	Request Message	9

4.2.2	Positive Respone Message	9

4.2.3	Supported Negative Response Codes	9

4.2.4	입출력 모니터링	9

4.2.5	Read Data Identifier of WPC	9

4.2.6	User Option Record Local Identifier – DID BE01	9

4.2.7	Input Output Record Local Identifier #1(Byte Data) – DID BE02	10

4.2.7.1	ES95486-02 New UDS (3세대 CAN 적용 시)	10

4.2.7.2	ES95486-00 UDS on CAN (2세대 CAN 적용 차종)	11

4.2.8	Input Output Record Local Identifier #2(Bit Data) - DID BE03	12

4.2.8.1	ES95486-02 New UDS (3세대 CAN 적용 시)	12

4.2.8.2	ES95486-00 UDS on CAN (2세대 CAN 적용 차종)	13

4.2.9	RDBI Read ECU Identification Service	14

4.2.9.1	F197 – System Name Or Engine Type DataIdentifier	14

4.2.9.2	F187 – Vehicle Manufacturer Spare Part Number DataIdentifier	14

4.2.9.3	F1B0 – ECU Software UNIT number Data Identifier	15

4.2.9.4	F1B1 – ECU Software UNIT1 Version Data Identifier	15

4.2.9.5	F18B – ECU Manufacturing Date DataIdentifier	16

4.2.9.6	F18C – ECU Serial Number Data Identifier	16

4.2.9.7	F191- Vehicle Manufacturer ECU Hardware Number DataIdentifier	16

4.2.9.8	F193- System Supplier ECU Hardware Version Number DataIdentifier 	17

4.2.9.9	F1A1 – ECU Supplier Code DataIdentifier	18

4.2.9.10	F100 – CAN Data Base Version Number DataIdentifier	18

4.2.9.11	F1EF - Read ECU Identification : LocalRXSWINDataIdentifier - DID #F1EF hex	18

4.2.9.12	F1C1 – ECUSoftwareUnit1IVDDataIdentifier	19

4.2.9.13	F17F – Platform Version Number DataIdentifier	19

4.2.9.14	F17D – Platform Hardware Version Number DataIdentifier	20

4.3	Input Output Control by Identifier (2F hex) Service	21

4.3.1	강제구동	21

4.3.2	IOCBI – Input Output Control by Local Identifier of WPC	21

4.4	Clear Diagnostic Information (14 hex) Service	22

4.4.1	Request Message	22

4.4.2	Positive Response Message	22

4.4.3	Supported Negative Response Codes	22

4.5	Read DTC information (19 hex) Service	22

4.5.1	Request Message	22

4.5.1.1	Positive Response Message	22

4.5.1.2	Supported Negative Response Codes	23

4.5.1.3	Status of DTC	23

4.6	DTC management	24

4.6.1	DTC Definition for WPC ECU	25

4.6.2	DTC for CAN Timeout	25

Table of Contents

Table 1: 용어 정의	5

Table 2: Data Identifier of WPC	9

Table 3: Supported Check Positive Response Message List	21

Table 4: Input Output Local Identifier	22

Table 5: CAN Communication DTC	25

Figure List

Figure 1: Present and Memorized failure relation	24

Figure 2: Present and Memorized failure Clear Case1	24

Figure 3: Present and Memorized failure Clear Case2	24

Figure 4: Standard ECUs CAN Timeout Detection	26

- 목적

이 문서는 WPC (Wireless Power Charger)의 진단 서비스에 대한 품질 만족도 보장을 위한 진단 사양을 정의한다.

- 적용범위

본 사양서는 WPC 진단 사양에 적용되는 진단 서비스와 요구사항에 대해 규정하며, 본 사양서와 도면 간 상이한 내용이 있을 경우 도면이 우선한다.

- 용어정의

| Item, abbreviation, acronym | Description |
| --- | --- |
| ACC | ACCessory |
| IGN1 | Ignition 1 |
| CAN | Controller Area Network |
| DIAG | DIAGnosis |
| DTC | Diagnostic Trouble Code |
| ECU | Electronic Control Unit |
| EMS | Engine Management System |
| FC | Function Code |
| GND | GrouND |
| ID | Identification Device |
| LF | Low Frequency |
| REQ | REQuest |
| RND | RaNDom |
| RSP | ReSPonse |
| SIL | Safety Integrity Level |
| SMK | SMartKey |
| SRX | Standalone Receiver |
| TP | TransPonder |
| <> | Relational operator meaning "Different than" |
| IGPM | Integrated Gateway and Power control Module |
| LDC | Low power Dc to dc Converter |
| UMS | User Mode Setting of IPM |
| CALIB | CALIBration data saving in EEPROM |
| Packet | RX가 TX로 메시지를 통신하는 데 이용하는 데이터 구조 |

Table 1: 용어 정의

DIAGNOSTIC SPECIFICATION

진단 사양은 HKMC ES95486-00 (UDS on CAN 진단통신 사양서)을 따른다.

Protocol Definition

진단 통신 서비스 지원 목록

|  | Req SID (HEX) | Diagnostic Modes |  |  |
| --- | --- | --- | --- | --- |
|  |  | Default Session (01h) | Programming Session (02h) | Extended Diagnostic Session (03h) |
| DiagnosticSessionControl | 10 | ● | ● | ● |
| SecurityAccess | 27 |  | ● | ● |
| StopDiagnosticSession | 20 | ● | ● | ● |
| ReadDataByIdentifier | 22 | ● |  | ● |
| Input Output Control | 2F |  |  | ● |
| ReadDTCInformation | 19 | ● |  | ● |
| ClearDiagnosticInformation | 14 | ● |  |  |
| RoutineControl | 31 |  |  | ● |
| RequestDownload | 34 |  | ● |  |
| RequestUpload | 35 |  | ● |  |
| TransferData | 36 |  | ● |  |
| RequestTransferExit | 37 |  |  |  |

Communication Parameters

| Name | Value | Description |
| --- | --- | --- |
| Bus Type | CAN | Body CAN |
| Bus Speed | Low Speed : 100kbps High Speed : 500kbps | CAN bus data rate (bps) |
| Physical Can ID Type | 11-Bit | Can ID Type for Physical requests |
| Physical Request CAN ID to WPC ECU | 0x725 (0x71E : 2세대) | Physical Request CAN identifier |
| Physical Response CAN ID from WPC ECU | 0x72D (0x73E : 2세대) | Physical Response CAN identifier |
| Physical Request CAN ID to RWPC ECU | 0x724 (0x71F : 2세대) | Physical Request CAN identifier |
| Physical Response CAN ID from RWPC ECU | 0x72C (0x73F : 2세대) | Physical Response CAN identifier |
| Physical Request CAN ID to WPC2 ECU | 0x723 | Physical Request CAN identifier |
| Physical Response CAN ID from WPC2 ECU | 0x72B | Physical Response CAN identifier |
| Functional Can Id Type | 11-Bit | Can Id Type for functional requests |
| Functional request CAN ID to WPC ECU | 0x7DF | Functional Request CAN identifier |

| 송수신 | ID | 송수신 형태 | Remark |
| --- | --- | --- | --- |
| REQUEST | 07DFh | 02 10 03 (02 10 90) |  |
| ANSWER | 073Eh | 02 50 90 |  |

표 5.1 Definition of Communication parameter

DiagnosticSessionControl (10 hex) Service

Diagnostic service that is not prescribed in this document follow to ES95486-00, ES95486-02.

SecurityAccess(27 hex) Service

The purpose of this service is to provide a means to access data and/or diagnostic services, which have restricted access for security, emissions, or safety reasons. The security concept uses a seed and key relationship. Diagnostic services which should use “Security Access” are described by “Diagnostic Serviec List” on ES95486-00, ES95486-02.

Request Message

Refer to ES95486-00, ES95486-02.

Positive Response Message

Refer to ES95486-00, ES95486-02.

Supported Negative Response Codes

Refer to ES95486-00, ES95486-02.

Security Algorithm (ASK not Applied)

Client requests the “Seed” to server. Server sends the “Seed” to client. Client sends the key which is calculated using “Security Algorithm” with seed and “Hex Values”. Server responeds that the “Key” was valid and that it will unlock itself. The “Security Algorithm” and “Hex Values” are below.

KEY = Security Algorithm(SEED)

Result 1 = KEY.Byte03 = ~(seed high byte.DataByte#3d)^(Hex Value #1)

Result 2 = KEY.Byte02 = ~(seed.DataByte#4d)^(Hex Value #2)

Result 3 = KEY.Byte01 = ~(seed.DataByte#5d)^(Hex Value #3)

Result 4 = KEY.Byte00 = ~(seed Low byte.DataByte#6d)^(Hex Value #4)

※ Hex Value #1 = 0x83, Hex Value #2 = 0xF7, Hex Value #3 = 0x83, Hex Value #4 = 0xF7

Figure 7: Security Access Message Flow Example - Server is in a 'Locked' State

Security Algorithm (ASK Applied)

ES95486-00(Revision 12th), ES95486-02(Revision 1st)의 '4.3 Security Access (27h) Service'를 만족해야 한다.

StopDiagnosticSession (20 hex) Service

Diagnostic service that is not prescribed in this document follow to ES95486-00, ES95486-02.

Read Data by Identifier (22 hex) Service

Request Message

Refer to ES95486-00, ES95486-02.

Dataidentifier Parameter F18B / F195 service must support function ID (07DF)

Positive Respone Message

Refer to ES95486-00, ES95486-02.

Supported Negative Response Codes

Refer to ES95486-00, ES95486-02.

입출력 모니터링

5.8.3.1.1 LED 출력 상태: AMBER 점등, GREEN 점등, AMBER 점멸

5.8.3.1.2 충전 전력: 충전 전력, 충전 전압, 충전 전류

5.8.3.1.3 충전 상태: OFF, 충전 중, 충전 완료

5.8.3.1.4 SMK FOB 써칭에 의한 충전 금지 신호 상태(L_LFSearhingOn_IN)

- 신호 ON: 충전 금지 상태

- 신호 OFF: 충전 가능 상태

5.8.3.1.5 전원 상태: B+ 입력상태

- 전원단 판단 관련하여 CAN통신 신호와 조합하지 않고 H/W 입력 받은 것에 대해서만 판단

Read Data Identifier of WPC

- Record Data Identifier of WPC

| Data Identifier | Description | Remark |
| --- | --- | --- |
| #BE01h | RDBI - User Option Record Local Identifier | Byte frame |
| #BE02h | RDBI - Input Output Record Local Identifier #1(Byte Data) | Byte frame |
| #BE03h | RDBI - Input Output Record Local Identifier #2(Bit Data) | Byte frame |
| #F1C0h | RDBI – ECUSecurityInformationDataIdentifier (Byte Data) | Byte frame |

Table 2: Data Identifier of WPC

User Option Record Local Identifier – DID BE01

- Request Message

|  | Message Byte | Hex Value | Parameter Name |
| --- | --- | --- | --- |
| Data Byte | #1 | 22 | ReadDataByIdentifier Service ID |
|  | #2 ~ #3 | XXXX | dataIdentifier - Input Output Record Local Identifier #1 |

- Positive Response Message

| Data Byte | PID | Hex Value | Parameter Name |
| --- | --- | --- | --- |
| #0 | - | 62 | Read Data By Local Identifier Request Service ID |
| #1~#2 | - | BE01 | Record Local Identifier (User Option) |

| Data Byte | PID | Hex Value | Description |  | Remark |
| --- | --- | --- | --- | --- | --- |
| #0 ~ #3 | 0x00 | XX | Supported PID | Supported PID : 0x01 ~ 0x20 | 0 = not supported, |
|  |  |  |  | Response : 4 Byte (A, B, C, D) | 1 = supported |
|  |  |  |  | Byte #0 : PID 0x01~0x08, Byte #1 : PID 0x09~0x10 |  |
|  |  |  |  | Byte #2 : PID 0x11~0x18, Byte #3 : PID 0x19~0x20 |  |

Input Output Record Local Identifier #1(Byte Data) – DID BE02

- Request Message

|  | Message Byte | Hex Value | Parameter Name |
| --- | --- | --- | --- |
| Data Byte | #1 | 22 | RearDataByIdentifier Service ID |
|  | #2~#3 | BE02 | dataIdentifier - Input Output Record Local Identifier #1 |

- Positive Response Message

| Data Byte | PID | Hex Value | Description | Remark |
| --- | --- | --- | --- | --- |
| #0 | - | 62 | Read Data By Local Identifier Response Service ID |  |
| #1~#2 | - | BE02 | Input Output Read #1 |  |

ES95486-02 New UDS (3세대 CAN 적용 시)

| Data Byte | PID | Hex Value | Description |  | Remark |
| --- | --- | --- | --- | --- | --- |
| #0 ~ #3 | 0x00 | XX | Supported PID | Supported PID : 0x01 ~ 0x20 | 0 = not supported, |
|  |  |  |  | Response : 4 Byte (A, B, C, D) | 1 = supported |
|  |  |  |  | Byte #0 : PID 0x01~0x08, Byte #1 : PID 0x09~0x10 |  |
|  |  |  |  | Byte #2 : PID 0x11~0x18, Byte #3 : PID 0x19~0x20 |  |
| #4 | 0x01 | XX | Input | Charging Voltage | 0.08 * X [V] |
| #5 | 0x02 | XX | Input | Charging Current | 0.0393 * X [A] |
| #6 | 0x03 | XX | Input | Charging Power | 0.196 * X [W] |
| #7 | 0x04 | XX | Input | BAT+ Voltage | 0.08 * X [V] |
| #8 | 0x05 | XX | Input | IGN1 Voltage | 0.08 * X [V] |
| #9 | 0x06 | XX | Input | ActiveCoilID | 1* X ( 0 ~ 2) Active Coil ID |
| #10 | 0x07 | XX | Input | Coil1 TempSensor1 | Unsigned Offset -40 1 * X (℃) (-40~85[℃]) |
| #11 | 0x08 | XX | Input | Coil2 TempSensor2 | Unsigned Offset -40 1 * X (℃) (-40~85[℃]) |
| #12 | 0x09 | XX | Input | Coil3 TempSensor3 | Unsigned Offset -40 1 * X (℃) (-40~85[℃]) |
| #13 | 0x0A | XX | Input | FAN Speed | [%] |
| #14 |  |  |  |  |  |
| #15 | 0x0C | XX | Input | WPC2 Charging Voltage | 0.08 * X [V] |
| #16 | 0x0D | XX | Input | WPC2 Charging Current | 0.0393 * X [A] |
| #17 | 0x0E | XX | Input | WPC2 Charging Power | 0.196 * X [W] |
| #18 | 0x0F | XX | Input | WPC2 Coil TempSensor | Unsigned Offset -40 1 * X (℃) (-40~85[℃]) |
| #19 | 0x10 | XX | Input | WPC2 FAN Speed | [%] |

※ ActiveCoilID 관련 케이스별 출력 조건

1) FOD에러: ERROR 발생 이전 충전 중이던 Coil ID 표출

2) 고온 에러: ‘0’표출

3) 과전류 에러: ‘0’표출

4) 온도센서 에러: ‘0’표출

5) IGN ON -> OFF 에 따른 미충전상태: 핑 출력중인 coil ID 표출

6) LF 서칭에 따른 미충전 상태: ‘0’표출

ES95486-00 UDS on CAN (2세대 CAN 적용 차종)

| Data Byte | PID | Hex Value | Description |  | Remark |
| --- | --- | --- | --- | --- | --- |
| #0 ~ #3 | 0x00 | XX | Supported PID | Supported PID : 0x01 ~ 0x20 | 0 = not supported, |
|  |  |  |  | Response : 4 Byte (A, B, C, D) | 1 = supported |
|  |  |  |  | Byte #0 : PID 0x01~0x08, Byte #1 : PID 0x09~0x10 |  |
|  |  |  |  | Byte #2 : PID 0x11~0x18, Byte #3 : PID 0x19~0x20 |  |
| #4 | 0x01 | XX | Input | Charging Voltage | 0.08 * X [V] |
| #5 | 0x02 | XX | Input | Charging Current | 0.0393 * X [A] |
| #6 | 0x03 | XX | Input | Charging Power | 0.196 * X [W] |
| #7 | 0x04 | XX | Input | BAT+ Voltage | 0.08 * X [V] |
| #8 | 0x05 | XX | Input | ACC Voltage | 0.08 * X [V] |
| #9 | 0x06 | XX | Input | ActiveCoilID | 1* X ( 0 ~ 2) Active Coil ID |
| #10 | 0x07 | XX | Input | Coil1 TempSensor1 | Unsigned Offset -40 1 * X (℃) (-40~85[℃]) |
| #11 | 0x08 | XX | Input | Coil2 TempSensor2 | Unsigned Offset -40 1 * X (℃) (-40~85[℃]) |
| #12 | 0x09 | XX | Input | Coil3 TempSensor3 | Unsigned Offset -40 1 * X (℃) (-40~85[℃]) |
| #13 | 0x0A | XX | Input | FAN Speed | [%] |
| #14 | 0x0B | XX | Input | IGN1 Voltage | 0.08 * X [V] |
| #15 | 0x0C | XX | Input | WPC2 Charging Voltage | 0.08 * X [V] |
| #16 | 0x0D | XX | Input | WPC2 Charging Current | 0.0393 * X [A] |
| #17 | 0x0E | XX | Input | WPC2 Charging Power | 0.196 * X [W] |
| #18 | 0x0F | XX | Input | WPC2 Coil TempSensor | Unsigned Offset -40 1 * X (℃) (-40~85[℃]) |
| #19 | 0x10 | XX | Input | WPC2 FAN Speed | [%] |

※ ActiveCoilID 관련 케이스별 출력 조건

1) FOD에러: ERROR 발생 이전 충전 중이던 Coil ID 표출

2) 고온 에러: ‘0’표출

3) 과전류 에러: ‘0’표출

4) 온도센서 에러: ‘0’표출

5) IGN ON -> OFF 에 따른 미충전상태: 핑 출력중인 coil ID 표출

6) LF 서칭에 따른 미충전 상태: ‘0’표출

Input Output Record Local Identifier #2(Bit Data) - DID BE03

- Request Message

|  | Message Byte | Hex Value | Parameter Name |
| --- | --- | --- | --- |
| Data Byte | #1 | 22 | RearDataByIdentifier Service ID |
|  | #2~#3 | BE03 | dataIdentifier - Input Output Record Local Identifier #2 |

- Positive Response Message

| Data Byte | PID | Hex Value | Description | Remark |
| --- | --- | --- | --- | --- |
| #0 | - | 62 | Read Data By Local Identifier Response Service ID |  |
| #1~#2 | - | BE03 | Input Output Read #2 |  |

ES95486-02 New UDS (3세대 CAN 적용 시)

| Data Byte | PID | Hex Value | Bit | Description |  | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| #0 ~ #3 | 0x00 | XX | - | Supported PID | Supported PID : 0x01 ~ 0x20 | 0 = not supported, |
|  |  |  |  |  | Response : 4 Byte (A, B, C, D) | 1 = supported |
|  |  |  |  |  | Byte #0 : PID 0x01~0x08, Byte #1 : PID 0x09~0x10 |  |
|  |  |  |  |  | Byte #2 : PID 0x11~0x18, Byte #3 : PID 0x19~0x20 |  |
| #4 | 0x01 | XX | B0 | Input | IGN1 | (0:OFF / 1:ON) |
|  | 0x02 |  | B1 | Input | Amber LED On/Off Status | (0:OFF / 1:ON) |
|  | 0x03 |  | B2 | Input | Amber Blinking Status | (0:OFF / 1:Blinking) |
|  | 0x04 |  | B3 | Input | Green LED On/Off Status | (0:OFF / 1:ON) |
|  | 0x05 |  | B4 | Input | SMK LF Searching Signal Status | (0:OFF / 1:ON) |
|  | 0x06 |  | B5 | Input | Charging Status | (0x00:Charging Off 0x01:Charging 0x02:Charging Complete 0x03:Invalid) |
|  | 0x07 |  | B6 | Input |  |  |
| #5 | 0x08 | XX | B0 | Input | Reserved | 0 |
|  | 0x09 |  | B1 | Input | WPC2 Amber LED On/Off Status | (0:OFF / 1:ON) |
|  | 0x0A |  | B2 | Input | WPC2 Amber Blinking Status | (0:OFF / 1:Blinking) |
|  | 0x0B |  | B3 | Input | WPC2 Green LED On/Off Status | (0:OFF / 1:ON) |
|  | 0x0C |  | B4 | Input | Reserved | 0 |
|  | 0x0D |  | B5 | Input | WPC2 Charging Status | (0x00:Charging Off 0x01:Charging 0x02:Charging Complete 0x03:Invalid) |
|  | 0x0E |  | B6 | Input |  |  |
| #6 | 0x0F |  | B0 B1 B2 | Input | WPC Charging mode | 0x00 : Default 0x01 : BPP 0x02 : SFC (Samsung fast Charging) 0x03 : EPP 0x04 : MPP 0x05~ 0x07 : Reserved |
| #7 | 0x12 |  | B0 B1 B2 | Input | WPC2 Charging mode | 0x00 : Default 0x01 : BPP 0x02 : SFC (Samsung fast Charging) 0x03 : EPP 0x04 : MPP 0x05~ 0x07 : Reserved |
| #8 | 0x15 |  | B0 | Input | WPC EPP Mode Authentication state | 0x0 : not auth 0x1 : auth |
| #9 | 0x16 |  | B0 | Input | WPC2 EPP Mode Authentication state | 0x0 : not auth 0x1 : auth |

ES95486-00 UDS on CAN (2세대 CAN 적용 차종)

| Data Byte | PID | Hex Value | Bit | Description |  | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| #0 ~ #3 | 0x00 | XX | - | Supported PID | Supported PID : 0x01 ~ 0x20 | 0 = not supported, |
|  |  |  |  |  | Response : 4 Byte (A, B, C, D) | 1 = supported |
|  |  |  |  |  | Byte #0 : PID 0x01~0x08, Byte #1 : PID 0x09~0x10 |  |
|  |  |  |  |  | Byte #2 : PID 0x11~0x18, Byte #3 : PID 0x19~0x20 |  |
| #4 | 0x01 | XX | B0 | Input | ACC | (0:OFF / 1:ON) |
|  | 0x02 |  | B1 | Input | Amber LED On/Off Status | (0:OFF / 1:ON) |
|  | 0x03 |  | B2 | Input | Amber Blinking Status | (0:OFF / 1:Blinking) |
|  | 0x04 |  | B3 | Input | Green LED On/Off Status | (0:OFF / 1:ON) Animation 동작시 ‘1:ON’ 으로 출력 |
|  | 0x05 |  | B4 | Input | SMK LF Searching Signal Status | (0:OFF / 1:ON) |
|  | 0x06 |  | B5 | Input | Charging Status | (0x00:Charging Off 0x01:Charging 0x02:Charging Complete 0x03:Invalid) |
|  | 0x07 |  | B6 | Input |  |  |
|  | 0x08 |  | B7 | Input | IGN1 | (0:OFF / 1:ON) |
| #5 | 0x09 | XX | B1 | Input | WPC2 Amber LED On/Off Status | (0:OFF / 1:ON) |
|  | 0x0A |  | B2 | Input | WPC2 Amber Blinking Status | (0:OFF / 1:Blinking) |
|  | 0x0B |  | B3 | Input | WPC2 Green LED On/Off Status | (0:OFF / 1:ON) |
|  | 0x0C |  | B4 | Input | Reserved | 0 |
|  | 0x0D |  | B5 | Input | WPC2 Charging Status | (0x00:Charging Off 0x01:Charging 0x02:Charging Complete 0x03:Invalid) |
|  | 0x0E |  | B6 | Input |  |  |
| #6 | 0x0F |  | B0 B1 B2 | Input | WPC Charging mode | 0x00 : Default 0x01 : BPP 0x02 : SFC (Samsung fast Charging) 0x03 : EPP 0x04 : MPP 0x05~ 0x07 : Reserved |
| #7 | 0x12 |  | B0 B1 B2 | Input | WPC2 Charging mode | 0x00 : Default 0x01 : BPP 0x02 : SFC (Samsung fast Charging) 0x03 : EPP 0x04 : MPP 0x05~ 0x07 : Reserved |
| #8 | 0x15 |  | B0 | Input | WPC EPP Mode Authentication state | 0x0 : not auth 0x1 : auth |
| #9 | 0x16 |  | B0 | Input | WPC2 EPP Mode Authentication state | 0x0 : not auth 0x1 : auth |

ECUSecurityInformationDataIdentifier(Byte Data) – DID F1C0

- Request Message

|  | Message Byte | Hex Value | Parameter Name |
| --- | --- | --- | --- |
| Data Byte | #1 | 22 | RearDataByIdentifier Service ID |
|  | #2~#3 | F1C0 | dataIdentifier - ECUSecurityInformationDataIdentifier |

- Positive Response Message

| Data Byte | PID | Hex Value | Description | Remark |
| --- | --- | --- | --- | --- |
| #0 | - | 62 | Read Data By Local Identifier Response Service ID |  |
| #1~#2 | - | F1C0 | ECU Security Information Read |  |

| Data Byte | PID | Hex Value | Description | Remark |
| --- | --- | --- | --- | --- |
| #0 | - | XX | HSM Configuration Lock State | 0 = Disabled 1 = Enabled |
| #1 | - | XX | Secure Boot State | 0 = Disabled 1 = Enabled |
| #2 | - | XX | Secure Debug State | 0 = Disabled 1 = Enabled 2 = DebugProtectionTempStop 3 = Debug Disable |

RDBI Read ECU Identification Service

| Request Message | Message Byte | Hex Value | Parameter Name |
| --- | --- | --- | --- |
| Data Byte | #1 | 22 | ReadEcuIdentification Request Service ID |
|  | #2~#3 | XXXX | DataIdentifier |

- 각각의 서비스 세션별 기능은 071Eh의 CAN Identifier로 REQUEST 시 응답한다.

단, DTC 개수(ReadDTCInformation, reportNumberOfDTCByStatusMask 01h),

제어기 제조일 (ReadDataByIdentifier, ECUManufacturingDateDataIdentifier, F18B),

S/W 버전(ReadDataByIdentifier, SoftwareVersionforHKMCVehicleManufactureDataIdentifier, F1A0 )은

07DF CAN Identifier로 REQUEST시에도 응답한다.

- Positive Response Message

F197 – System Name Or Engine Type DataIdentifier

|  | Message Byte | Hex Value | Parameter Name |
| --- | --- | --- | --- |
| Data Byte | #1 | 62 | readEcuIdentification Positive Response Service ID |
|  | #2 ~ #3 | F187 | System Name |
|  | #4 ~ #7 |  | dataRecord[] #1 = [ |
|  |  |  | data#1 |
|  |  |  | ... |
|  |  |  | data#7] [ASCII] |

- Notation --- System Name :

- EX)

- System Name : WPC [ASCII]

F187 – Vehicle Manufacturer Spare Part Number DataIdentifier

|  | Message Byte | Hex Value | Parameter Name |
| --- | --- | --- | --- |
| Data Byte | #1 | 62 | readEcuIdentification Positive Response Service ID |
|  | #2 ~ #3 | F187 | Vehicle Part Number |
|  | #4 ~ #13 |  | dataRecord[] #1 = [ |
|  |  |  | data#1 |
|  |  |  | ... |
|  |  |  | data#10] [ASCII] |

- Notation --- Vehice Part Number :

F1B0 – ECU Software UNIT number Data Identifier

ECUSoftwareUNITnumberDataIdentifier

This value shall be used to reference the number of S/W Unit in the ECU(server). The number of S/W Unit must consist of within 1byte and data type is unit8.  If there is only one S/W unit, it is expressed as 0x1. (Increase the hex value by 1 according to the number of S/W units.), and It can be expressed up to 0xA. DID starts from F1B1 and be increased by 1.

e.g) The number of SW unit is 3 then only three DIDs are allowed to use as ECUSoftwareUNITVersionDataIdentifier F1B1, F1B2, F1B3.

Request Message

| Data Byte | Parameter Name | Cvt | Hex Value |
| --- | --- | --- | --- |
| #1 | ReadDataByIdentifier Request Service ID | M | 22 |
| #2 #3 | dataIdentifier[]#1 = [ byte#1 (MSB) byte#2 ] | M M | F1B0 |

Positive Response Message

| Data Byte | Parameter Name | Cvt | Hex Value |
| --- | --- | --- | --- |
| #1 | ReadDataByIdentifier Response Service ID | M | 62 |
| #2 #3 | dataIdentifier[]#1 = [ byte#1 (MSB) byte#2 ] | M M | F1B0 |
| #4 | dataRecord[] #1 = [ data #1] | M | [HEX] |

- Notation ---  The number of S/W Unit :

F1B1 – ECU Software UNIT1 Version Data Identifier

This value shall be used to reference the ECU S/W UNIT 1 version. It should be expressed by ASCII code.

Request Message

| Data Byte | Parameter Name | Cvt | ASCII Value |
| --- | --- | --- | --- |
| #1 | ReadDataByIdentifier Request Service ID | M | 22 |
| #2 #3 | dataIdentifier[]#1 = [ byte#1 (MSB) byte#2 ] | M M | F1B1 |

Positive Response Message

| Data Byte | Parameter Name | Cvt | Hex Value |
| --- | --- | --- | --- |
| #1 | ReadDataByIdentifier Response Service ID | M | 62 |
| #2 #3 | dataIdentifier[]#1 = [ byte#1 (MSB) byte#2 ] | M M | F1B1 |
| #4 - #8 | dataRecord[] #1 = [ data#1 … data#5] | M | [ASCII] Ex) |

- Notation --- Supplier S/W Unit1 Version :

- Ex)

Data Byte #4d = 0x32(‘2’)

Data Byte #5d = 0x34(‘4’)

Data Byte #6d = 0x38(‘8’)

Data Byte #7d = 0x31(‘1’)

Data Byte #8d = 0x55(‘U’)

차종 버전 (5Byte)

24 8 1 U

- ②  ③  ④

① 소프트웨어 배포 연, ‘0~9’ 만 사용 가능

ex) 24년 배포 : 24

② 소프트웨어 배포 월, ‘1~9’, A,B,C 만 사용 가능

ex) 8월 배포 : 8  /  10월 배포 : A

③ 소프트웨어 연월의 배포 차수, ‘1~9’, ‘A~Z’, ‘a~z’ 사용 가능 (순서 : 숫자<대문자<소문자)

ex) 24년 8월 2번째 배포시 : 2

④ OTA 평가용 소프트웨어 SUMS 등록 이력관리

ex) 배포버전 : 0 / U-ROM 개발 : U / Fail-ROM 개발 : F / R-ROM 개발 : R

F18B – ECU Manufacturing Date DataIdentifier

|  | Message Byte | Hex Value | Parameter Name |
| --- | --- | --- | --- |
| Data Byte | #1 | 62 | readEcuIdentification Positive Response Service Id |
|  | #2 ~ #3 | F18B | Supplier H/W Manufacturing Date |
|  | #4 ~ #7 |  | dataRecord[] #1 = [ |
|  |  |  | data#1 |
|  |  |  | ... |
|  |  |  | data#4] [HEX] |

- Notation --- Supplier H/W Manufacturing Date :

- EX) 2016년 1월 31일 생산 → 62 F1 8B 20 16 01 31

→ 10진수 숫자를 그대로 16진수로 표현

F18C – ECU Serial Number Data Identifier

|  | Message Byte | Hex Value | Parameter Name |
| --- | --- | --- | --- |
| Data Byte | #1 | 62 | readEcuIdentification Positive Response Service Id |
|  | #2 ~ #3 | F18C | Serial Number |
|  | #4 ~ #18 |  | dataRecord[] #1 = [ |
|  |  |  | data#1 |
|  |  |  | ... |
|  |  |  | data#15] [ASCII] |

- Notation  ---  Serial Number :

F191- Vehicle Manufacturer ECU Hardware Number DataIdentifier

This value is HKMC defined ECU hardware version number shown on the drwaing of each system. H/W version shall be defined as ASCII code.

Request Message

| Data Byte | Parameter Name | Cvt | Hex Value |
| --- | --- | --- | --- |
| #1 | ReadDataByIdentifier Request Service ID | M | 22 |
| #2 #3 | dataIdentifier[]#1 = [ byte#1 (MSB) byte#2 ] | M M | F191 |

Positive Response Message

| Data Byte | Parameter Name | Cvt | Hex Value |
| --- | --- | --- | --- |
| #1 | ReadDataByIdentifier Response Service ID | M | 62 |
| #2 #3 | dataIdentifier[]#1 = [ byte#1 (MSB) byte#2 ] | M M | F191 |
| #4 - #7 | dataRecord[] #1 = [ vehicle ManufacturerECUHardwareNumberData] | M | Ex) |

- Notation --- HKMC H/W Version :

- Ex)

Data Byte #4d = 0x31(‘1’)

Data Byte #5d = 0x2E(‘.’)

Data Byte #6d = 0x30(‘0’)

Data Byte #7d = 0x30(‘0’)

F193- System Supplier ECU Hardware Version Number DataIdentifier

This value shall be used to reference the system supplier specific ECU (server) hardware version number. Record data content and format shall be server specific and defined by the system supplier.

Request Message

| Data Byte | Parameter Name | Cvt | Hex Value |
| --- | --- | --- | --- |
| #1 | ReadDataByIdentifier Request Service ID | M | 22 |
| #2 #3 | dataIdentifier[]#1 = [ byte#1 (MSB) byte#2 ] | M M | F193 |

Positive Response Message

| Data Byte | Parameter Name | Cvt | Hex Value |
| --- | --- | --- | --- |
| #1 | ReadDataByIdentifier Response Service ID | M | 62 |
| #2 #3 | dataIdentifier[]#1 = [ byte#1 (MSB) byte#2 ] | M M | F193 |
| #4 - #7 | dataRecord[] #1 = [ SystemSupplierECUHardwareVersionNumberData] | M | [ASCII] Ex) |

- Notation --- Supplier H/W Version :

- Ex)

Data Byte #4d = 0x31(‘1’)

Data Byte #5d = 0x2E(‘.’)

Data Byte #6d = 0x30(‘0’)

Data Byte #7d = 0x30(‘0’)

F1A1 – ECU Supplier Code DataIdentifier

This value shall be used to reference the ECU (server) Supplier Code.

Record data format shall be ASCII and size shall be 4bytes.

(e.g. DELPHI CORPORATION : AB9H -> 0x41 42 39 48)

Request Message

| Data Byte | Parameter Name | Cvt | Hex Value |
| --- | --- | --- | --- |
| #1 | ReadDataByIdentifier Request Service ID | M | 22 |
| #2 #3 | dataIdentifier[]#1 = [ byte#1 (MSB) byte#2 ] | M M | F1A1 |

Positive Response Message

| Data Byte | Parameter Name | Cvt | Hex Value |
| --- | --- | --- | --- |
| #1 | ReadDataByIdentifier Response Service ID | M | 62 |
| #2 #3 | dataIdentifier[]#1 = [ byte#1 (MSB) byte#2 ] | M M | F1A1 |
| #4 - #7 | dataRecord[] #1 = [ data#1 ... data#4] | M | [ASCII] |

- Notation --- System Supplier Code :

F100 – CAN Data Base Version Number DataIdentifier

| F100 - CANDataBaseVersionNumberDataIdentifier |  |  |  |
| --- | --- | --- | --- |
|  | Message Byte | Hex Value | Parameter Name |
| Data Byte | #1 | 62 | readEcuIdentification Positive Response Service Id |
|  | #2 ~ #3 | F100 | CANDataBaseVersionNumberDataIdentifier |
|  | #4 ~ #7 |  | dataRecord[] #1 = [ |
|  |  |  | data #1 |
|  |  |  | ... |
|  |  |  | data#4] |

Ex)

Data Byte #4d = 0x31(‘1’)

Data Byte #5d = 0x2E(‘.’)

Data Byte #6d = 0x30(‘0’)

Data Byte #7d = 0x30(‘0’)

F1EF - Read ECU Identification : LocalRXSWINDataIdentifier - DID #F1EF hex

| Data Byte | Parameter Name | Cvt | Hex Value | Mnemonic |
| --- | --- | --- | --- | --- |
| #1d | ReadDataByIdentifier Response Service ID | M | #62h | RDBIPR |
| #2d #3d | DataIdentifier #1 = [ byte#1(MSB) byte#2(LSB) ] | M M | #F1EFh | DID_ HB LB |
| #4d #5d … #35d | DataRecord[] #1 = [ LocalRXSWINInfo ] | M M M | #XXh #XXh … #XXh | DREC_ DATA_1 DATA_2 … DATA_32 [ASCII] |

Positive Response Message Definition - LocalRXSWINDataIdentifier (#F1EF hex)

Refer to ES95486-00(Annex J) or ES95486-02(Annex J) for more detail requirements.

F1C1 – ECUSoftwareUnit1IVDDataIdentifier

This value shall be used to reference the IVD value of SW unit 1. Each unit shall have own IVD value to prove SW integrity. This value has to be same with IVD which is in the SUMS.

| Data Byte | Parameter Name | Cvt | Hex Value | Mnemonic |
| --- | --- | --- | --- | --- |
| #1d | ReadDataByIdentifier Response Service ID | M | #62h | RDBIPR |
| #2d #3d | DataIdentifier #1 = [ byte#1(MSB) byte#2(LSB) ] | M M | #F1C1h | DID_ HB LB |
| #4d #5d … #35d | DataRecord[] #1 = [ ECUSoftwareUnit1IVDDataIdentifier ] | M M M | #XXh #XXh … #XXh | DREC_ DATA_1 DATA_2 … DATA_32 [HEX] |

F17F – Platform Version Number DataIdentifier

| F17F – PlatformVersionNumberDataIdentifier |  |  |  |
| --- | --- | --- | --- |
|  | Message Byte | Hex Value | Parameter Name |
| Data Byte | #1 | 62 | readEcuIdentification Positive Response Service Id |
|  | #2 ~ #3 | F17F | Platform Version |
|  | #4 ~ #17 | [ASCII] | dataRecord[] #1 = [ |
|  |  |  | data #1 |
|  |  |  | ... |
|  |  |  | data#11] |

플랫폼 버전

01.00.00.00.00

①     ②     ③     ④      ⑤

- 플랫폼 사양서(차수) 버전

ex) 12차수 버전 반영시: 12.00.00.00

- CAN (표준) 버전

ex) B-HSCAN  또는 B-CANFD의 표준 버전 0.3 반영시: 12.03.00.00

③ Local-CAN (표준) 버전

※ SV 이전: 00, SV 이후 관리 (B-CANFD 변경 시점 연동)

④ OTA 호환성 버전: 같은 TYPE내 OTA호환성 구분 기준: A, B, C,…

※ 신규 TYPE 부터 반영, 기존 TYPE은 S/W & H/W 변경이력서에 업데이트 관리

⑤ 플랫폼 자체 S/W 문제 개선 업데이트 시

ex) 플랫폼 버전 12.03.00.00에서 S/W 구현 오류 개선시 12.03.00.01

※ ‘⑤’ 버전 이력은 ‘①’플랫폼 사양서 버전 업데이트 시 초기화됨

F17D – Platform Hardware Version Number DataIdentifier

| F17F – PlatformVersionNumberDataIdentifier |  |  |  |
| --- | --- | --- | --- |
|  | Message Byte | Hex Value | Parameter Name |
| Data Byte | #1 | 62 | readEcuIdentification Positive Response Service Id |
|  | #2 ~ #3 | F17F | Platform Hardware Version |
|  | #4 ~ #14 | [ASCII] | dataRecord[] #1 = [ |
|  |  |  | data #1 |
|  |  |  | ... |
|  |  |  | data#11] |

플랫폼 버전

01.00.00.00

①        ②        ③       ④

- 플랫폼 타입

ex) 타입 02 반영시 : 02.00.00.00

- 능동 소자 변경시

ex) MCU 사양, TR, DIODE등 소자 변경시 : 02.01.00.00

- 수동소자 변경시

ex) Resistor, Capacitor, Inductor 소자 변경시: 02.01.01.00

- PCB 패턴 변경시

ex) PCB내 Via 및 경로등 수정시: 02.01.01. 01

ED90 – Non Sleep Collected Each Ecu DataIdentifier

ED90 – NonSleepCollectedEachEcuDataIdentifier

| PID | 저장 데이터 | Size (Byte) | Byte | Bit | Description | Conversion rule |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | 미슬립 조건 Index | 1 | #0 | bit 0 | FD_B2 CAN | 0x0 : Off 0x1 : On |
|  |  |  |  | bit 1 | Local CAN |  |
|  |  |  |  | bit 2 | Local CAN 2 (Reserved) |  |
|  |  |  |  | bit 3 | ECU 내부 조건 |  |
|  |  |  |  | bit 4 | HW Input |  |
|  |  |  |  | bit 5 | Network 조건 |  |
|  |  |  |  | bit 6 | (Reserved) |  |
|  |  |  |  | bit 7 | (Reserved) |  |
| $02 | FD_B2 CAN | 4 | #0 | bit 0 | ACC | 0x0 : Off 0x1 : On |
|  |  |  |  | bit 1 | IGN1 |  |
|  |  |  |  | bit 2 | DrvDrSw |  |
|  |  |  |  | bit 3 | AstDrSw |  |
|  |  |  |  | bit 4 | RLDrSw |  |
|  |  |  |  | bit 5 | RRDrSw |  |
|  |  |  |  | bit 6 | AutobrightValue |  |
|  |  |  |  | bit 7 | SMKOption |  |
|  |  |  | #1 | bit 0 | (Reserved) |  |
|  |  |  |  | bit 1 | (Reserved) |  |
|  |  |  |  | bit 2 | (Reserved) |  |
|  |  |  |  | bit 3 | (Reserved) |  |
|  |  |  |  | bit 4 | (Reserved) |  |
|  |  |  |  | bit 5 | (Reserved) |  |
|  |  |  |  | bit 6 | (Reserved) |  |
|  |  |  |  | bit 7 | (Reserved) |  |
|  |  |  | #2 | bit 0 | ComM Mode PNC153 | 0 : PNC Requested 1 : PNC Ready Sleep 2 : PNC Prepare Sleep 3 : PNC No Communication 4 : PNC Full Communication |
|  |  |  |  | bit 1 |  |  |
|  |  |  |  | bit 2 |  |  |
|  |  |  |  | bit 3 | ComM Mode PNC141 |  |
|  |  |  |  | bit 4 |  |  |
|  |  |  |  | bit 5 |  |  |
|  |  |  |  | bit 6 | (Reserved) |  |
|  |  |  |  | bit 7 | (Reserved) |  |
|  |  |  | #3 | bit 0 | (Reserved) |  |
|  |  |  |  | bit 1 | (Reserved) |  |
|  |  |  |  | bit 2 | (Reserved) |  |
|  |  |  |  | bit 3 | (Reserved) |  |
|  |  |  |  | bit 4 | (Reserved) |  |
|  |  |  |  | bit 5 | (Reserved) |  |
|  |  |  |  | bit 6 | (Reserved) |  |
|  |  |  |  | bit 7 | (Reserved) |  |
| $03 | LOCAL CAN | 4 | #0 | bit 0 | OwnerPhnRegRVal | 0x0 : Off(inactive) 0x1 : On(active) |
|  |  |  |  | bit 1 | OPAdv_On_Cmd |  |
|  |  |  |  | bit 2 | WPD_Device_On_Cmd |  |
|  |  |  |  | bit 3 | (Reserved) |  |
|  |  |  |  | bit 4 | (Reserved) |  |
|  |  |  |  | bit 5 | (Reserved) |  |
|  |  |  |  | bit 6 | (Reserved) |  |
|  |  |  |  | bit 7 | (Reserved) |  |
|  |  |  | #1 | bit 0 | ComM Mode PNC120 | 0 : PNC Requested 1 : PNC Ready Sleep 2 : PNC Prepare Sleep 3 : PNC No Communication 4 : PNC Full Communication |
|  |  |  |  | bit 1 |  |  |
|  |  |  |  | bit 2 |  |  |
|  |  |  |  | bit 5 | (Reserved) |  |
|  |  |  |  | bit 6 | (Reserved) |  |
|  |  |  |  | bit 7 | (Reserved) |  |
| $04 | LOCAL CAN 2 (Reserved) | 4 | #0 | bit 0 | (Reserved) |  |
|  |  |  |  | bit 1 | (Reserved) |  |
|  |  |  |  | bit 2 | (Reserved) |  |
|  |  |  |  | bit 3 | (Reserved) |  |
|  |  |  |  | bit 4 | (Reserved) |  |
|  |  |  |  | bit 5 | (Reserved) |  |
|  |  |  |  | bit 6 | (Reserved) |  |
|  |  |  |  | bit 7 | (Reserved) |  |
|  |  |  | #1 | bit 0 | (Reserved) |  |
|  |  |  |  | bit 1 | (Reserved) |  |
|  |  |  |  | bit 2 | (Reserved) |  |
|  |  |  |  | bit 3 | (Reserved) |  |
|  |  |  |  | bit 4 | (Reserved) |  |
|  |  |  |  | bit 5 | (Reserved) |  |
|  |  |  |  | bit 6 | (Reserved) |  |
|  |  |  |  | bit 7 | (Reserved) |  |
| $05 | ECU 내부 데이터 | 6 | #0 | bit 0 | UDS LP Condition | 0 : logic is in High Power Mode 1 : logic is in Low Power Mode |
|  |  |  |  | bit 1 | WPC LP Condition |  |
|  |  |  |  | bit 2 | WCT LP Condition |  |
|  |  |  |  | bit 3 | NFC LP Condition |  |
|  |  |  |  | bit 4 | (Reserved) |  |
|  |  |  |  | bit 5 | (Reserved) |  |
|  |  |  |  | bit 6 | (Reserved) |  |
|  |  |  |  | bit 7 | (Reserved) |  |
|  |  |  | #1 | bit 0 | (Reserved) |  |
|  |  |  |  | bit 1 | (Reserved) |  |
|  |  |  |  | bit 2 | (Reserved) |  |
|  |  |  |  | bit 3 | (Reserved) |  |
|  |  |  |  | bit 4 | (Reserved) |  |
|  |  |  |  | bit 5 | (Reserved) |  |
|  |  |  |  | bit 6 | (Reserved) |  |
|  |  |  |  | bit 7 | (Reserved) |  |
|  |  |  | #2 | bit 0 | (Reserved) |  |
|  |  |  |  | bit 1 | (Reserved) |  |
|  |  |  |  | bit 2 | (Reserved) |  |
|  |  |  |  | bit 3 | (Reserved) |  |
|  |  |  |  | bit 4 | (Reserved) |  |
|  |  |  |  | bit 5 | (Reserved) |  |
|  |  |  |  | bit 6 | (Reserved) |  |
|  |  |  |  | bit 7 | (Reserved) |  |
|  |  |  | #3 | bit 0 | (Reserved) |  |
|  |  |  |  | bit 1 | (Reserved) |  |
|  |  |  |  | bit 2 | (Reserved) |  |
|  |  |  |  | bit 3 | (Reserved) |  |
|  |  |  |  | bit 4 | (Reserved) |  |
|  |  |  |  | bit 5 | (Reserved) |  |
|  |  |  |  | bit 6 | (Reserved) |  |
|  |  |  |  | bit 7 | (Reserved) |  |
| $06 | HW Input | 1 | #0 | bit 0 | A_IGN1_IN | 0x0 : Off / 0x1 : On |
|  |  |  |  | bit 1 | (Reserved) |  |
|  |  |  |  | bit 2 | (Reserved) |  |
|  |  |  |  | bit 3 | (Reserved) |  |
|  |  |  |  | bit 4 | (Reserved) |  |
|  |  |  |  | bit 5 | (Reserved) |  |
|  |  |  |  | bit 6 | (Reserved) |  |
|  |  |  |  | bit 7 | (Reserved) |  |
| $07 | Network 조건 | 1 | #0 | bit 0 | Diag Msg | 0x0 : Off(inactive) 0x1 : On(active) |
|  |  |  |  | bit 1 | Autosar NM PNC On |  |
|  |  |  |  | bit 2 | CAN Wake up |  |
|  |  |  |  | bit 3 | Local CAN Wake up |  |
|  |  |  |  | bit 4 | Local CAN 2 Wake up (Reserved) |  |
|  |  |  |  | bit 5 | (Reserved) |  |
|  |  |  |  | bit 6 | (Reserved) |  |
|  |  |  |  | bit 7 | (Reserved) |  |

Input Output Control by Identifier (2F hex) Service

강제구동

5.4.1.1 LED 강제 구동(AMBER, GREEN 각각): 1초 ON, 1초 OFF 주기로 5회 점멸

구동 이후 정지 명령시 즉시 정지

5.4.1.2 전원 조건: IGN1 이상

IOCBI – Supported Input Output Local Identifier

|  |  | Bit7 | Bit6 | Bit5 | Bit4 | Bit3 | Bit2 | Bit1 | Bit0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DID 0xF010 | Supported DID Data_A | Unused | Unused | Unused | Unused | Unused | Unused | Unused | Unused |
|  | Supported DID Data_B | Unused | Unused | Unused | Unused | Unused | Unused | Unused | Unused |
|  | Supported DID Data_C | DID0xXX11 | DID0xXX12 | DID0xXX13 | DID0xXX14 | DID0xXX15 | DID0xXX16 | DID0xXX17 | DID0xXX18 |
|  | Supported DID Data_D | DID0xXX19 | DID0xXX1A | DID0xXX1B | DID0xXX1C | DID0xXX1D | DID0xXX1E | DID0xXX1F | DID0xXX20 |
| DID 0xF020 | Supported DID Data_A | DID0xXX21 | DID0xXX22 | DID0xXX23 | DID0xXX24 | DID0xXX25 | DID0xXX26 | DID0xXX27 | DID0xXX28 |
|  | Supported DID Data_B | DID0xXX29 | DID0xXX2A | DID0xXX2B | DID0xXX2C | DID0xXX2D | DID0xXX2E | DID0xXX2F | DID0xXX30 |
|  | Supported DID Data_C | DID0xXX31 | DID0xXX32 | DID0xXX33 | DID0xXX34 | DID0xXX35 | DID0xXX36 | DID0xXX37 | DID0xXX38 |
|  | Supported DID Data_D | DID0xXX39 | DID0xXX3A | DID0xXX3B | DID0xXX3C | DID0xXX3D | DID0xXX3E | DID0xXX3F | DID0xXX40 |
| DID 0xF040 | Supported DID Data_A | DID0xXX41 | DID0xXX42 | DID0xXX43 | DID0xXX44 | DID0xXX45 | DID0xXX46 | DID0xXX47 | DID0xXX48 |
|  | Supported DID Data_B | DID0xXX49 | DID0xXX4A | DID0xXX4B | DID0xXX4C | DID0xXX4D | DID0xXX4E | DID0xXX4F | DID0xXX50 |
|  | Supported DID Data_C | DID0xXX51 | DID0xXX52 | DID0xXX53 | DID0xXX54 | DID0xXX55 | DID0xXX56 | DID0xXX57 | DID0xXX58 |
|  | Supported DID Data_D | DID0xXX59 | DID0xXX5A | DID0xXX5B | DID0xXX5C | DID0xXX5D | DID0xXX5E | DID0xXX5F | DID0xXX60 |

- Request Message (Supported Check를 위한 요구 명령)

|  | Message Byte | Hex Value | Parameter Name |
| --- | --- | --- | --- |
| Data Byte | #1 | 2F | InputOutputControlByIdentifier Request Service ID |
|  | #2 ~ #3 | XXXX | dataIdentifier |
|  | #4 | XX | controlOptionRecord ※ Supported check control Parameter 0x02 |

- Supported Check Positive Response Message

|  | Message Byte | Hex Value | Parameter Name |
| --- | --- | --- | --- |
| Data Byte | #1 | 6F | Positive Response Identification |
|  | #2 ~ #3 | XXXX | dataIdentifier - Supported Input Output Identifier |
|  | #4 | 02 | controlOptionRecord |
|  | #5 | XX | Supported DID #1 |
|  | #6 | XX | Supported DID #2 |
|  | #7 | XX | Supported DID #3 |
|  | #8 | XX | Supported DID #4 |

Table 3: Supported Check Positive Response Message List

IOCBI – Input Output Control by Local Identifier of WPC

- Request Message (선택된 해당 항목에 대한 구동 명령)

|  | Message Byte | Hex Value | Parameter Name |
| --- | --- | --- | --- |
| Data Byte | #1 | 2F | Request Service Identifier |
|  | #2 ~ #3 | F0XX | Input Output Local Identifier |
|  |  |  | (refer to Input Output Local Identifier table) |
|  | #4 | XX | Input Output Control Parameter |
|  |  |  | - 0x00: STOP(returnControlToECU) |
|  |  |  | - 0x03: START(shortTermAdjustment) |

| Input/ Output | Control By Local Identifier | Output | Output Time | Remark |
| --- | --- | --- | --- | --- |
|  |  | Condition |  |  |
| Hex Value | Parameter Description |  |  |  |
| F000 | Supported check DID F000 |  |  |  |
| F011 | AMBER LED | IGN1 | 1s ON, 1s Off 5회 반복 | IOCtrl Parameter 0x00: STOP (returnControlToECU) 명령시 즉시 정지 듀얼WPC의 경우 UNIT 1/2 동시 동작 |
| F012 | GREEN LED | IGN1 | 1s ON, 1s Off 5회 반복 | IOCtrl Parameter 0x00: STOP (returnControlToECU) 명령시 즉시 정지 LED 3개 적용사양은 3개 LED 모두 출력 듀얼WPC의 경우 UNIT 1/2 동시 동작 |
| F013 | FAN ON | IGN1 | 50% DUTY 5s 출력 | 듀얼WPC의 경우 UNIT 1/2 동시 동작 |
| $F014 | FAN OFF | IGN1 | OFF 후 OFF 상태 유지 | 듀얼WPC의 경우 UNIT 1/2 동시 동작 |
| $F015 | FAN ON 10 | IGN1 | 10% DUTY 지속 출력 | 듀얼WPC의 경우 UNIT 1/2 동시 동작 |
| $F016 | FAN ON 20 | IGN1 | 20% DUTY 지속 출력 | 듀얼WPC의 경우 UNIT 1/2 동시 동작 |
| $F017 | FAN ON 30 | IGN1 | 30% DUTY 지속 출력 | 듀얼WPC의 경우 UNIT 1/2 동시 동작 |
| $F018 | FAN ON 40 | IGN1 | 40% DUTY 지속출력 | 듀얼WPC의 경우 UNIT 1/2 동시 동작 |
| $F019 | FAN ON 50 | IGN1 | 50% DUTY 지속 출력 | 듀얼WPC의 경우 UNIT 1/2 동시 동작 |
| $F01A | FAN ON 60 | IGN1 | 60% DUTY 지속 출력 | 듀얼WPC의 경우 UNIT 1/2 동시 동작 |
| $F01B | FAN ON 70 | IGN1 | 70% DUTY 지속 출력 | 듀얼WPC의 경우 UNIT 1/2 동시 동작 |
| $F01C | FAN ON 80 | IGN1 | 80% DUTY 지속 출력 | 듀얼WPC의 경우 UNIT 1/2 동시 동작 |
| $F01D | FAN ON 90 | IGN1 | 90% DUTY 지속 출력 | 듀얼WPC의 경우 UNIT 1/2 동시 동작 |
| $F01E | FAN ON 100 | IGN1 | 100% DUTY 지속 출력 | 듀얼WPC의 경우 UNIT 1/2 동시 동작 |

Table 4: Input Output Local Identifier

Clear Diagnostic Information (14 hex) Service

Request Message

Refer to ES95486-00, ES95486-02.

Positive Response Message

Refer to ES95486-00, ES95486-02.

Supported Negative Response Codes

Refer to ES95486-00, ES95486-02.

Read DTC information (19 hex) Service

Request Message

- Request Message : Report Number of DTC by Status Mask

Refer to ES95486-00, ES95486-02.

*Report number of DTC by status mask service must support function ID (07DF)

- Request Message : Report DTC by Status Mask

Refer to ES95486-00, ES95486-02.

- Request Message : Report DTC Extended Data Record By DTC Number

Refer to ES95486-00, ES95486-02.

Request message definitions for sub-function that is not prescribed in this document follow to ES95486-00, ES95486-02. It is recommended that ECU supports sub-functions marked as 'Mandatory' on the sub-function table. Refer to ES95486-00, ES95486-02.

Positive Response Message

- Positive Response Message : Report DTC Extended Data Record By DTC Number

Refer to ES95486-00, ES95486-02.

- Positive Response Message : Report DTC by Status Mask

Refer to ES95486-00, ES95486-02.

The server shall perform a bit-wise logical AND-ing operation between the mask specified in the client's request and the actual status associated with each DTC supported by the server. In addition to the DTCStatusAvailabilityMask, the server shall return all DTCs for which the result of the AND-ing operation is non-zero (i.e., (statusOfDTC & DTCStatusMask) != 0).

Example : If client sends request message (sub-function : Report DTC bt Status Mask) with DTC Status Mask = 0x0C, server sends positive response message with DTCs which bit 3 or bit 2 of its 'status of DTC' is '1' (regradless of other bits).

DTCStatusAvailabilityMask : A byte whose bits are defined the same as statusOfDTC and represents the status bits that are supported by the server. Bits that are not supported by the server shall be set to '0'. Each supported bit (indicated by a value of '1') shall be implemented for every DTC supported by the server. For example, a ECU supports the status bit0, 1 and 7, then DTCStatusAvailabilityMask = #89h regardless of request message and status of DTCs.

- Positive Response Message : Report DTC Extended Data Record By DTC Number

Refer to ES95486-00, ES95486-02.

Positive response message definitions for sub-function that is not prescribed in this document follow to ES95486-00, ES95486-02.

Supported Negative Response Codes

Refer to ES95486-00, ES95486-02.

Status of DTC

Refer to “DTC Status bit definition” on ES95486-00, ES95486-02.

DTC management

1) Set DTC Control

When Error is detected, Present failure and Memorized failure are set same time.

Figure 1: Present and Memorized failure relation

2) Clear DTC Control

Case1:

At the time to receive the command "Clear DTC service", Present failure and Memorized failure are cleared same time.

Figure 2: Present and Memorized failure Clear Case1

Case2:

If the failure condition is not found during 30(thirty) times of IGN1 ON/OFF

cycle after memorizing corresponding DTC then clear the memorized DTC.

Figure 3: Present and Memorized failure Clear Case2

DTC Definition for WPC ECU

| DTC | EC | Trouble description | Set Condition | Detecting Time | DTC Memory | Reset Condition | Recovery Time | 충전 여부 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B1602-00 |  | CAN error | CAN Transceiver ERR pin 이 low로 발생 시 | 2000ms | O | CAN Transceiver ERR pin 이 High로 발생 시 | 50ms | 가능 |
| B1603-00 |  | CAN Bus Off | CAN High/Low 양단 Short 또는 BAT+/GND Short 발생 시 | Consecutive bus off counter가 2일 때 | O | CAN 정상 통신 시 | 1000ms | 가능 |
| B160F-00 |  | IAU_Local-CAN Bus Off | CAN High/Low 양단 Short 또는 BAT+/GND Short 발생 시 | Consecutive bus off counter가 2일 때 | O | CAN 정상 통신 시 | 1000ms | 가능 |
| B1621-00 |  | Temperature Sensor Fault | 하나 이상 온도센서 단선/단락 | 2000ms | X | IGN1 Off -> On | X | 금지 |
|  |  | FAN Fault | FAN 이상 발생시 | 2000ms | X | IGN1 Off -> On | X | 가능 |
|  |  | WPC ECU Internal Error | 충전부 (Charging Control MCU, Gate Driver, V/I Sensor, Demodulation…) 이상 발생 시 | 2000ms | X | 충전부 정상 복귀 시 | 500ms | 금지 |
| B1621-01 |  | Temperature Sensor Fault | 하나 이상 온도센서 단선/단락 | 2000ms | X | IGN1 Off -> On | X | 금지 |
| B1621-02 |  | FAN Fault | 하나 이상의 FAN 이상 발생시 | 2000ms | X | IGN1 Off -> On | X | 가능 |
| B1621-03 |  | WPC ECU Internal Error | 충전부 (Charging Control MCU, Gate Driver, V/I Sensor, Demodulation…) 이상 발생 시 | 2000ms | X | 충전부 정상 복귀 시 | 500ms | 금지 |

Table 5: CAN Communication DTC

※ B1602 DTC는 저속 B-CAN에만 적용되며, 고속 B-CAN에서는 적용되지 않는다.

B1621-00 (ECU Hardware Error) DTC 는3-COIL 사양에 적용한다.

B1621-01/02/03 DTC 는 1-COIL 사양에 적용한다.

DTC for CAN Timeout

1) BASIC concept of DTC for CAN timeout

(1) Timeout checking Condition:

A) The WPC shall detect ECU's CAN timeout by Network Management messages (periodic)

when Timeout detection is enabled from network active until network shutdown.

(2) Timeout Set Condition:

A) The WPC records the CAN timeout DTC (present & memorized) when IGN(A_IGN1 == On ||

L_IGN2 == On) is On condition and Timeout is detected.

B) 1 sec delay after IGN(A_IGN1 == On || L_IGN2 == On) ON shall be applied before starting

Confirmation Timer

Figure 4: Standard ECUs CAN Timeout Detection